MM 226 Supervised Learning project consent form

PART II _ Submission

Name of project supervisor(s):  Prof. Prasanna Kumar S. Mural

Title of project: AI-driven Smart Gloves for Motion Detection

Project participant details: (Max 5 participants)

| Roll Number | Name of Student | Role of participant in project |
| --- | --- | --- |
| 21D110016 | M. Hassan Shaikh | Project Lead & AI Researcher |
| 21D110012 | Om Kulpe | Prototype Engineer & Software Developer |
| 21D110007 | Keloth Uday Krishna | Documenter & Prototype Engineer |
| 210110054 | Jai priyadarshi | AI Researcher & Experimentation |
| 210110038 | Dev Kumar acharya | Experimentation & Software Developer |

Objectives of the project: the deliverables of the project must be clearly outlined.

Develop smart gloves equipped with piezoelectric and triboelectric sensors to detect hand motion & gestures for sign language recognition

1. Designing & 3D printing Polyvinylidene fluoride polymer based triboelectric highly shape adaptive sensor for sensitive joint motion monitoring & tactile sensing, demonstrating it’s application in real-time human-machine interaction

2. Utilizing an Arduino micro-controller to develop a custom sketch program file that interface with a piezoelectric/triboelectric 3D printed sensor, enabling precise real-time signal recording, advanced processing and data logging for motion detection

3. Pre-Processing the signal using advanced Signal Processing techniques, apply moving window mechanism to construct input matrix and implement LSTM/Transformer/RNNs/ other hybrid models to achieve a proposed accuracy.

Role of each member:

We have decided the roles on basis of what individual wants to learn and hence allotted multiple roles(so it follows as primary & secondary roles).

Responsibility of each roles are as follows:

Project Lead

- Communicating with the project supervisor and TA regarding the status of the project

- Arranging lab visits for experimentation and resolving queries

- Arranging materials required for the project and budget management

Documenter

- Documenting the contents discussed in meetings for future reference

- Creating reports and presentations involved in the project

Prototype Engineer

- Assembling all components to build prototypes of the gloves

- Overlooking integration of the software and hardware involved

Experimentation

- Mechanical testing and structural analysis of materials used

- Electrical testing of microcontroller (Arduino)

- Establishing connection between the software and the electrical components

AI Researcher

- Researching about efficient techniques for motion detection

- Building and testing different AI models

Software Developer

- Building a program to detect and analyse signals coming from the glove

- Applying AI models to classify hand gestures

Timeline of the project (in the format given below)

| Month | Week | Assigned task |
| --- | --- | --- |
| August | I | - Brainstorming ideas - Setting objectives - Finalising project details |
|  | II |  |
|  | III | - Processing polymer to generate filament - 3D printing components - Mechanical testing and structural analysis |
|  | IV |  |
| September | I | Integration of all physical components of the smart glove |
|  | II |  |
|  | III | MIDSEM |
|  | IV |  |
| October | I | - Software development and AI model building - Connecting software to hardware - Testing and modifying |
|  | II |  |
|  | III |  |
|  | IV |  |
| November | I | Ideating and planning the usage of this technology in other domains |
|  | II |  |
|  | III | ENDSEM |
|  | IV |  |

Mid- semester progress

1] Key achievements thus far

Mode Selection:

- Explored two sensing modes: contact-separation and linear-sliding.

- Choose linear-sliding mode for voltage generation based on triboelectric motion detection.

A setup to test triboelectric sensor with multilayer sliding configuration

Material Selection:

- Evaluated different materials for triboelectric sensors, settling on PVDF, PLA, nylon and  ABS.

3D printed PLA strips                     3D printed Nylon strips                      3D printed ABS strips

Sensor Structure Design:

- Finalized the structural design of the triboelectric sensors through team discussions.

- Created 3D models of the sensor structures using Blender software for precise fabrication.

- Designed a multi-layer structure to accompany the triboelectric behavior according to linear sliding configuration.

3D Printing of Sensors:

- Produced PVDF filaments in the polymer processing lab for sensor fabrication.

- Utilized Flashforge Guider IIs 3D printer to print multiple polymer strips(as discussed above) for testing the triboelectric effect.

Component Procurement:

- Ordered essential electronics components, including Arduino microcontrollers, breadboards, and nylon gloves via Amazon.

- Purchased additional components (copper tape, resistors, lead wires, USB cable, multimeter) from local electronics stores.

Sensor Assembly:

- Attached copper electrodes to the sensors using copper tape for effective signal conduction.

- Connected sensors to the multimeter using lead wires for voltage measurement.

Testing and Measurement:

- Tested the sensors by rubbing positive triboelectric materials (nylon) over negative triboelectric materials (PLA, ABS).

- Achieved measurable voltage outputs in the range of 2.0x200-4.0x200 mV, indicating successful triboelectric charge generation.

2] Challenges faced and solved

PVDF Filament Moisture capture

- During 3D printing, the PVDF filament exhibited a blackish color due to moisture exposure.

- To address this, we planned to remade the filament and implemented a storage solution using moisture-free containers with silica gel.

Nylon Sensor Printing Issues

- Initial attempts to 3D print nylon sensors were unsuccessful due to material compatibility issues with the printer.

- Through experimentation, we determined the optimal printing temperature to be 230°C, slightly above nylon's melting point (220°C), resolving structural inconsistencies that arose at lower (230°C) and higher (250°C) temperatures.

at 230°C                                                                          at 250°C(over melt)

Sensor Structure Design

- We tested three designs for the sensor: (a) long rectangular strips, (b) rectangular strips joined with zig-zag springs, and (c) rectangular strips joined with solenoid-shaped springs.

- The first design lacked aesthetics, and the second was difficult to assemble due to the need for additional materials.

- We chose the third solenoid design for its simplicity and structural viability.

Sensor Attachment Method

- Initially, we used double-sided tape to attach sensors to the glove, but it added unnecessary insulation.

- We switched to wrapping Kapton tape around the sensor and glove fingers, providing a more suitable attachment method without compromising performance.

- We are yet to assemble this but have tested without it.

Triboelectric Sensing Model

- We compared two sensing modes: contact separation and linear sliding.

- Contact separation required complex mechanical assembly, so we opted for the linear sliding mode, which offered a simpler and more feasible implementation.

3] If any changes in the project, then clearly mention the new objectives.

There are no changes in the project. We have already achieved one of our objectives, that is,

- Designing & 3D printing Polyvinylidene fluoride polymer based triboelectric highly shape adaptive sensor for sensitive joint motion monitoring & tactile sensing, demonstrating it’s application in real-time human-machine interaction

The remaining objectives are

- Utilizing an Arduino micro-controller to develop a custom sketch program file that interface with a piezoelectric/triboelectric 3D printed sensor, enabling precise real-time signal recording, advanced processing and data logging for motion detection

- Pre-Processing the signal using advanced Signal Processing techniques, apply moving window mechanism to construct input matrix and implement LSTM/Transformer/RNNs/ other hybrid models to achieve a proposed accuracy

Timeline – Part II Submission

The parts highlighted in green have been completed. We are right on track as per our initial timeline.

| Month | Week | Assigned task |
| --- | --- | --- |
| August | I | Brainstorming ideas Setting objectives Finalising project details |
|  | II |  |
|  | III | Processing polymer to generate filament 3D printing components Mechanical testing and structural analysis |
|  | IV |  |
| September | I | Integration of all physical components of the smart glove(sensor assemble) |
|  | II |  |
|  | III | MIDSEM |
|  | IV |  |
| October | I | Software development and AI model building Connecting software to hardware Testing and modifying |
|  | II |  |
|  | III |  |
|  | IV |  |
| November | I | Ideating and planning the usage of this technology in other domains |
|  | II |  |
|  | III | ENDSEM |